TZ-NVR10CA

Features

- Supports MIPI display interface, 6-pin capacitive touch interface

- Supports dual storage via SD card and hard drive

- Supports external speaker output with audio-visual synchronization

- Supports external microphone input for intercom communication with cameras

- Supports online upgrades for NVR and cameras

- Supports multi-channel remote monitoring via mobile and PC clients for anytime, anywhere viewing

Specifications

| Module |  |
| --- | --- |
| Model | TZ-NVR10CA |
| Video/Audio Input |  |
| IP Video Input | 10-ch |
| Video/Audio Output |  |
| HDMI/RGB Output | 1 channel, Resolution: 1024x768, 1280x1024, 1440x900, 1920x1080, 4K@30Hz with audio output support |
| Recording Resolution | 8MP/5MP/3MP/1080P/720P |
| Audio/Video Decoding Format | G.711A |
| Decoding |  |
| Video Processing | H.265/H.264 |
| Synchronous Playback | 1080P: 4 channels @4MP / 2 channels @5MP / 1 channel @8MP 4K: 4 channels @ 3MP / 2 channels @ 5MP / 1 channel @ 8MP |
| Recording Management |  |
| Recording Mode | Scheduled Recording, Motion Detection Recording |
| Playback Mode | Motion Recording, Scheduled Recording, Human Detection Recording |
| Backup | USB Flash Drive Backup / App Playback Point Backup |
| Storage |  |
| SD Card | 1 MicroSD Slot, Supports up to 256GB SD Card / Cloud Storage |
| Hard Drive | 1 hard drive interface, supports up to 8TB capacity / Cloud storage |
| Power |  |
| Charging Voltage | DC 12V 2A |
| Interfaces |  |
| Internal WIFI Antenna | 2 x 2.4GHz/5.8GHz paired WIFI antenna ports |
| Internet WIFI Antenna | 1 x 2.4GHz/5.8GHz internet WIFI antenna port |
| Network Interface | 1 x RJ45 10M/100M auto-negotiating Ethernet port |
| USB Ports | 1 USB 2.0 port |
| Operating Environment |  |
| Operating Temperature | -20°C to 50°C |
| Indicator Lights |  |
| Power Light | Steady blue light when powered on |
| Recording Light HDD | Slow flash: Recording in progress, 1 flash per second Steady blue: Storage device normal, waiting for recording Off: No storage device detected (SD card or HDD) |
| Operation LED | Slow blue flash: Device operating normally Off: Device malfunction |
| Specifications |  |
| Dimensions (with packaging) | 32 × 27.7 × 6 cm |
| Color | White |