TZ-NVR16A

Features

16-channel HD wireless surveillance deployment

Supports dual-band WiFi 4 and 6 with concurrent operation to reduce co-channel interference

Automatic wireless pairing and device addition; supports up to 8MP cameras

Mixed-model compatibility for diverse surveillance angle requirements

One-click auto-cascading eliminates manual setup, enabling effortless wireless relay to extend coverage range

H.265 high compression ratio reduces storage space and transmission bandwidth

Built-in wireless network card eliminates cable constraints

Integrated high-sensitivity microphone ensures audio-visual synchronization

Zone motion detection with real-time alert notifications

Supports multi-channel remote monitoring via mobile/PC clients for anytime, anywhere access

Specifications

| Module |  |
| --- | --- |
| Model | TZ-NVR16A |
| Video/Audio Input |  |
| Network Video/Audio Input | 16 channels, 8MP |
| Video/Audio Output |  |
| HDMI Output | 1 channel, Resolution: 1024x768, 1280x1024, 1440x900, 1920x1080, 4K@30Hz with audio output support |
| VGA Output | 1 channel, Resolution: 1024x768, 1280x1024, 1440x900, 1920x1080p |
| Video Decoding Standard |  |
| Video Decoding | H.264/H.265 |
| Audio Decoding Format |  |
| Audio Decoding | G711A / AAC |
| Video/Audio Codec Parameters |  |
| Recording Resolution | 8MP/5MP/3MP/1080P/720P |
| Simultaneous Playback | 4 channels @ 5MP; 2 channels @ 8MP |
| Recording Management |  |
| Recording Mode | Scheduled Recording, Motion Detection Recording |
| Playback Mode | Smart Playback, Standard Playback, Event Playback |
| Backup | USB Flash Drive Backup |
| Hard Drive |  |
| Type | 1 SATA Interface |
| Maximum Capacity | Supports cloud storage or hard drives up to 8TB capacity |
| External Interfaces |  |
| Network Port | 1 RJ45 10M/100M auto-negotiating Ethernet port, built-in wireless functionality |
| Antenna Ports | 3 total: 2 external 2.4G/5.8G antennas, 1 internal 2.4G antenna |
| Network Port | 1 RJ45 10M/100M auto-negotiating Ethernet port, built-in wireless functionality |
| USB Ports | 1 USB 2.0 port |
| Audio Ports | 1x 3.5mm headphone jack (optional speaker amplifier output) |
| General Specifications |  |
| Power Supply | DC: 12V 2A |
| Device Power Consumption (excluding hard drive) | ≤5W |
| Operating Temperature | -20°C to 50°C, humidity <95% (non-condensing) |
| Startup Temperature | -20°C to 50°C, humidity <95% (non-condensing) |
| Storage Temperature | -40°C to 80°C, humidity <95% (non-condensing) |
| Interface Type |  |
| Power Supply | DC12V2A |
| Light Board Interface | 5-pin 2.54mm (1 port) for light board (NET network light, REC recording light, ground, 3.3V power, coding key) |
| Reset Button | 1 reset button. Press and hold for 3 seconds to perform a simple factory reset (clear password). Press and hold for 10 seconds to perform a full factory reset (delete all camera channels) |
| Internal WiFi Antenna | 2 paired WiFi antenna ports |
| External WiFi Antenna | 1 Internet WiFi antenna port |
| VGA Port | 1 VGA port |
| HDMI Port | 1 HDMI port |
| USB Port | 1 USB 2.0 port |
| MIC Port | 1 x 2-pin MIC jack |
| Speaker jack | 1 x 2-pin speaker jack |

Function Module Block Diagram