SP1 Low Power Consumption - 4MP (WIFI Version)

WTD904

Features

Smart Features, Human-shaped detection

Wake-up Methods, PIR motion detection wake-up, Remote active wake-up

Alarm Notification, Mobile app push notifications, device white light flashing, and audible alarm

Pan/Tilt Range, Horizontal 350°, Vertical 90°

Specifications

| Model | WTD904 |
| --- | --- |
| Image |  |
| Sensor | 1/3" CMOS, 4 megapixels |
| Lens Focal Length | 3 mm |
| Lens Aperture | F1.6 |
| Field of View | Diagonal 135°, Horizontal 112°, Vertical 60° |
| Minimum Illumination | Color: 0.1Lux@F2.3, Monochrome: 0.01Lux@F2.3 |
| Compression | AAC/G711 |
| Audio |  |
| Input | Built-in microphone |
| Output | Built-in speaker |
| Two-Way Intercom | Full-duplex |
| Compression | AAC/G711 |
| Video |  |
| Compression | H.264/H.265 |
| Frame Rate | 15fps |
| Maximum image resolution | 2560*1440 |
| Night vision light source | Infrared lamp or white light lamp |
| Network |  |
| Wireless Standard | IEEE 802.11 b/g/n/a/ac/ax |
| Wireless WiFi | Supports 2.4 GHz & 5.8 GHz (Dual-Band) |
| Wired Network | Not supported |
| Recording |  |
| Recording Method | Alarm-triggered recording, manual recording |
| Recording Storage | Cloud storage, Local Micro SD card (supports up to 128 GB) |
| Indicator Lights |  |
| LED Indicators | Red and blue lights indicating power/network signal/function status |
| APP |  |
| APP | Eseecloud |
| Battery |  |
| Capacity | 5200 mAh |
| Removability | Non-removable |
| Operating Time | 3 months (recording 20 x 10-second videos daily) Note: Actual battery life may vary |
| Standby Time | 10 months Note: Actual battery performance may vary |
| Charging Voltage | 5 V ±5% |
| Charging Current | 1.5 A |
| Charging Time | 8 hours Note: Actual battery performance may vary |
| Protection Mechanisms | Overcharge, over-discharge, overcurrent, short circuit, and high/low temperature charging protection |
| Power Consumption |  |
| Standby | 200uA/0.76 mW |
| Daytime Operation | 0.76W |
| Nighttime Operation | 1.6W |
| Buttons | 1 Power Button, 1 Reset Hole |
| Interfaces |  |
| Power | TYPE-C |
| Storage | Micro SD Card |
| Button | Reset Button |
| Installation |  |
| Mounting | Wall-mounted, Ceiling-mounted |
| Bracket Specifications | 1/4'' Universal Screw Holes |
| Environment |  |
| Dust and Water Resistance | IP65 |
| Operating Temperature | -10 ℃ ~ 45 ℃ |
| Storage Temperature | -20°C ~ 60°C |
| Specifications |  |
| Dimensions | 140 mm × 95 mm × 140 mm (with bracket) |
| Unit Weight | Approx. 470 g |
| Material | ABS+PC |
| Color | White |
| Certifications |  |
| Certifications | CE, FCC, RoHS, WEEE, MSDS, UN38.3 |